CHANDIGARH ENGINEERING COLLEGE JHANJERI
Department of Computer Science & Engineering / Robotics and Artificial Intelligence
Affiliated to I.K. Gujral Punjab Technical University, Jalandhar (Batch: 2023–2027)

PROJECT SYNOPSIS
EDGE AI SMART PROSTHETIC HAND WITH ADAPTIVE GRIP LEARNING

1. Introduction & Clinical Motivation

Upper-limb amputations severely impair manual manipulation and quality of life. Commercial myoelectric hands rely primarily on open-loop threshold switching, which lacks tactile feedback and causes high rates of object slippage and crush damage, while imposing extreme visual concentration on users. This project presents an Edge AI-enabled prosthetic hand simulation prototype that decouples intention recognition from autonomous closed-loop tactile force regulation within a sub-2 ms host latency profile.

2. Problem Statement & Quantifiable Objectives

Problem Statement: Existing myoelectric prostheses lack low-latency, embedded-compatible pattern recognition combined with autonomous closed-loop tactile force regulation, leading to frequent object dropping, accidental crush damage, and amputee cognitive exhaustion.

Key Objectives: 1) Synthesize deterministic 2-channel sEMG signals (1000 Hz, 4 grip classes). 2) Extract 12 guarded time-domain features. 3) Train a compact Random Forest classifier (< 200 KB). 4) Profile real host CPU latency. 5) Implement closed-loop tactile force tracking (50 N target, 5 N contact threshold, 3 N deadband). 6) Implement an autonomous +18% anti-slip reflex boost. 7) Enforce a 75 N anti-crush safety ceiling. 8) Expose FastAPI REST endpoints with HTTP 422 validation. 9) Build a responsive High-DPI React 19 dashboard. 10) Provide an automated 27-second continuous demonstration. 11) Maintain 100% pass rate across 27 automated regression tests.

3. System Architecture & Methodology

The prototype implements a four-tier architecture: 1) Sensory Simulation Tier (1000 Hz sEMG synthesis from FDS and EDC muscles with Gaussian noise calibration); 2) Edge Intelligence Tier (12D feature extraction and 35-tree Random Forest inference); 3) Closed-Loop Tactile Control Tier (proportional force regulation, deadband hysteresis, +18% slip reflex boost, and 75 N crush ceiling); and 4) Telemetry Presentation Tier (React 19 / TypeScript dashboard with High-DPI canvas oscilloscope and 3D kinematic hand model).

4. Empirical Results & Verification Evidence

1) Automated Regression Tests: 27 out of 27 automated pytest tests passing in 4.06 seconds. 2) Production Build: Verified with Vite (13 modules transformed, 0 errors, 669 ms). 3) Classification Accuracy: 100.0% holdout accuracy and 100.0% 5-fold cross-validation accuracy (Macro-F1: 1.0000) on the synthetic dataset (disclaimer: evaluated on synthetic sEMG, not human clinical data). 4) Host Inference Latency: Measured mean latency of 1.46 ms +/- 0.08 ms across 50 warmed-up trials on host CPU. 5) Closed-Loop Regulation: Confirmed deadband stabilization at 49.8 N, +18% anti-slip boost within 20 ms, and 75 N safety ceiling clamping.

5. Five-Phase Hardware Transition Roadmap

Phase 1: TinyML C/C++ translation and benchmarking on ESP32-S3 (240 MHz). Phase 2: Analog Front-End (AFE) PCB design with TI INA333 instrumentation amplifiers and Ag/AgCl dry electrodes. Phase 3: 3D-printed anthropomorphic hand fabrication using PETG/TPU and coreless metal-gear micro-servos. Phase 4: Integration of fingertip piezoresistive FSR sensors and micro-vibration slip accelerometers. Phase 5: Standardized clinical amputee trials (SHAP, Box & Blocks).